Slide #1

Reporter: Head of Mines Rescue Services Department of Emercom of Russia

Member of the International Academy of Ecology and Life Protection Sciences,

Dr. Alexander Sin

Name of the report: Development of Emergency Prevention and Response Plan

For the last 15 years, the statistics on emergencies and incidents at coal mines in Russia has shown, that as a rule accidents occur in workings, where coal-face works or development works are being proceeded.

Herewith 90 % of all accidents accrue to the fires and explosions, which are more harmful for health and safety of the miners and mine rescuers, they cause large property loss and have the most labor-intensive recovery process.

Each mine has its own emergency response plan in order to eliminate emergency at the initial stage, prevent its further development and to rescue people, but it is focused only on elimination of emergency consequences.

Increase of the mining intensity significantly increase the risk of emergency occurrence.

Taking into consideration that it is easier to prevent any emergency than to eliminate it, today the emergency response planning should be primarily focused on prevention of any emergency situation that can occur and only after it on its elimination.

Slide #2

The existing practice of carrying out mine rescue operations at initial stage of emergency helped to indicate following serious disadvantages of the currently applied emergency response plans. Here are some of them:

- substantive provision of the Emergency Response Plan and ventilation scheme contains a lot of preplanned response procedures, made for the workings where emergencies practically never happen, moreover it contains a lot of single-type actions repeated nearly in every preplanned response procedure;

- another disadvantage is that it is very difficult to choose the right preplanned emergency procedure of the Emergency Response Plan in short period of time if there are no accurate information about the location of fire or explosion.

- Also to identify the volume of mine roadways being gassed often becomes impossible in case of explosion in the mine. At the initial stage this matter decrease operational efficiency of mine rescue teams arrived for examination of the emergency place in order to provide first aid to the injured miners;

- and the main disadvantage is that Emergency Response Plan is aimed to carry out the actions that can help to decrease potential consequences of emergency situation, but doesn’t include actions that could prevent an emergency itself.

- Slide #3

A lot of emergency situations develop during several working shifts, days and even weeks, that is why operations superintendent needs a document, that will allow to control dangerous areas in the mine, equipment and machineries in real-time mode.

For this purpose we suggest to apply “Emergency Prevention and Elimination Plan”. The fundamental part of this document is devoted to the approach of identification of potential hazards.

Slide #4

In slide you can see suggested structure of Emergency Prevention and Response Plan. It consists of a text part, drawings and documents.

The text part of Emergency Prevention and Response Plan includes:

- substantive provision;

Substantive provision of the Emergency Prevention and Elimination Plan consists of preplanned emergency procedures and Plans of prognosis of rock conditions in longwall and drifting faces applied to each planned emergency procedure.

- Each preplanned emergency procedure includes responsibilities of people, who take part in emergency response and the order of their actions, the list of officials, who should be immediately notified in case of emergency, the code of conduct at emergency situation for mine staff and guidelines for elimination of consequences.

Every emergency procedure consists of two parts Liquidating and Preventive.

Slide # 5

The Preventive part of the emergency procedure consists of two sections: Informative and Operational

Informative part reflects potential risks that may occur during operations and that can turn into emergency situation, for example zones of excessive fissuring.

Moreover Informative part has columns where accumulates such information as: concentration of methane, rate of face advance, dustiness, risk of endogenous fire and etc.

Operational part is worked out according to the equipment used in workings, and contains inspection data of shearers and continuous heading machines, advancing support, conveyors, compressors, pumps and other equipment.

Slide #6

In slide you can see Plan of prognosis of rock conditions in coal - face that consists of conveyor drift and ventilation face with applied pickets that are connected with the lines in a reticulate way.

Problem areas of coal face such as zones of excessive fissuring identified by surveying and during drifting works, are marked in the Plan of prognosis of rock conditions and recorded in corresponding column, in line B, of Informative section.

Filling in of the Plan of prognosis of rock conditions and corresponding column of Informative section is done by surveying division of the mine.

Slide #7

Filling in of the Preventive part of the preplanned emergency procedure is done by a mine foreman. Before the mine foreman starts his work he learns information recorded in the Preventive part of the preplanned emergency procedure by the mine foreman of the previous shift and gets as we call it “The Work order for the shift” with a control form printed on its back-page. The control form is filled in during the shift. At the end of the working shift the mine foreman transfer all data from the control blank to the Preventive part of the preplanned emergency procedure and signs it up.

The control form should be filed into a special folder and kept for at least 3 months.

Slide #8

In slide you can see suggested form of control form.

So in this way every engineering employee who is responsible for safety at the mine has an opportunity to analize information recorded at any period at any working area and to develop emergency prevention activities according to this data.

Information of the Informative section of Preventive part of the preplanned emergency procedure allows to determine design and technical mistakes that were made during the planning and mining.

Slide #9

Drawings of the Emergency Prevention and Response Plan include ventilation schemes, layouts of mine workings, surface plan, plans of mining works by the strata and levels.

On the ventilation schemes coal –face workings, development face and workings where belt conveyors are operated and where the reversal ventilation will be applied in case of fire breaking out are marked with color.

From the end of every preplanned emergency procedure along the direction of the return ventilation air up to the exit to the surface colored lines are drawn on the Ventilation scheme.

Workings where coal-face works and development works are not done, belt conveyors are not operated are not colored.

Slide #10

Additionally to this symbols red circles are drawn on the Ventilation Scheme, that show alarm sensors, which are installed in the mine roadways:

- Near the exits to the surface;

- At the places of air split;

- At different points of extended parallel routes;

- At the beginning and at the end of the heading where there is a risk of hazards;

- At the points where reverse ventilation ends;

Alarm sensors can be installed freestanding or as a part of automated control complex.

The main requirement to the alarm sensors is ability to transmit data to dispatcher's control desk in real-time mode concerning carbon monoxide concentration in mine air.

Liquidating part of the preplanned emergency procedure has one section and contains actions aimed to eliminate occurred accident and information concerning route direction of mines rescue teams and their actions, and also data on specific operations that are needed to be done for emergency liquidation.

Slide #11

Those actions, that are contained in every preplanned emergency procedure of the Emergency Prevention and Response plan are taken out to one document “Common arrangements of Emergency Prevention and Response Plan”.

This document is the main guide for dispatcher in case of fire or explosion occurrence.

Slide #12

Suggested form of the Emergency Prevention and Response Plan allows to reduce the time for taking right decision and to bring into action suitable preplanned emergency procedure, accumulates information that helps to make detailed evaluation and identify problems that can cause an occurrence of emergency situation, helps to make workers of the mine deal with safety issues on a regular basis, and what is really important - that helps to identify pre-emergency situations in advance and prevent occurrence of its potential consequences.

Slide #13

Thank you for attention!